# 1. Overview

The Xeryon SDK is a cross-platform C++ library designed to provide a unified interface for controlling Xeryon controllers across multiple hardware families and communication protocols.

The SDK abstracts controller differences and communication mechanisms while exposing a consistent motion control API to application developers.

Supported controller families include (but are not limited to):

- XD-OEM

- XD-M

- XD-C

- Future Xeryon controllers

# 2. Design Goals

## Primary Goals

- One SDK for all controllers

- Simple and clear public API

- Hardware abstraction

- Support single-axis and multi-axis controllers

- Cross-platform compatibility

- Protocol independence

- Transport independence

- Minimal runtime overhead

## Secondary Goals

- Easy extensibility

- Stable API surface

- Clean architecture

- Minimal dependencies

- logging support (Optional)

# 3. Non-Goals

The SDK does NOT:

- Expose low-level hardware registers

- Depend on platform-specific APIs

- Require GUI frameworks

- Bind the API to a specific controller model

- Embed protocol details in public interfaces

# 4. High-Level Architecture

The SDK follows a layered architecture:

[Diagram: Xeryon SDK Layered Architecture]

# 5. Core Concepts

## 5.1 Axis

An Axis represents a controllable motion channel.

An axis abstraction exists independently of controller topology:

- single-axis controller → one axis

- multi-axis controller → multiple axes

The Axis abstraction hides hardware complexity from users.

[Diagram: Controller and Axis Relationship]

## 5.2 Controller (SDK Abstraction)

A controller represents a physical hardware device.

Responsibilities:

- manage axes

- translate commands

- expose controller capabilities

- handle device-specific behavior

Examples:

- Controller ctrl("COM3", 115200)

Controllers are interchangeable implementations beneath a common interface.

## 5.3 Command Channel

CommandChannel is a single unified interface for sending commands and receiving responses from the device, independent of transport (Serial, SPI, I2C, etc.) or protocol (ASCII, binary, etc.).

It sits between the control layer (Axis, Controller) and the communication stack, so higher-level code does not need to know how data is actually transmitted.

It provides two core operations:

- send(cmd) → send a command without expecting a response

- query(cmd) → send a command and get a response

Internally, it ensures thread-safe access and delegates actual communication to the underlying protocol and transport layers.

## 5.4 INFO (Realtime Feedback)

INFO enables realtime controller feedback.

Used for:

- position updates

- motion state reporting

- diagnostics

- monitoring

Design characteristics:

- asynchronous reception

- non-blocking operation

- event-driven internally

- optional consumption by application

INFO handling is independent of transport type.

# 6. Layer Breakdown

## 6.1 Public API Layer

Provides user-facing interfaces:

Axis

Controller

Distance

Example:

Controller ctrl("COM3", 115200);
Axis& axis = ctrl.axis();

axis.setDPOS(5_mm);

axis.index(1);

The public API does not expose protocols or transports.

## 6.2 Axis Layer

Responsibilities:

- motion operations

- state queries

- settings access

Axis converts user actions into commands via controller.

[Diagram: Command to Hardware Communication Flow]

## 6.3 Command Channel

Responsibilities:

- Provides a single API for device communication (send, query)

- Ensures thread-safe command execution

- Forwards commands to the underlying protocol layer (e.g., ASCII, binary)

- Handles request/response transactions for queries

- Abstracts away transport details (serial, SPI, I2C, etc. via protocol stack)

- Acts as a synchronization point for concurrent command access

- Provides consistent logging of outgoing commands

- Decouples higher-level logic (Controller, Axis) from communication implementation

## 6.4 Protocol Layer

The protocol layer is the bridge between raw transport (serial/USB/etc.) and structured communication (commands + streaming data).

It ensures that bytes in/out become meaningful system events.

Examples:

- ASCII protocol

- Other protocol (Future scope)

Responsibilities:

- Converts raw transport I/O into structured ASCII lines

- Runs a reader thread that continuously reads device output

- Separates command responses vs streaming info data

- Matches query responses with pending commands

- Routes non-command data to InfoListener

- Handles thread-safe send/query operations

- Implements timeout-based request/response handling

- Normalizes protocol keys for matching (CMD, AXIS:CMD, etc.)

## 6.5 Transport Layer

The Transport layer is the lowest communication layer responsible for raw byte exchange with hardware devices over a physical or virtual medium (e.g. Serial/USB).

It is completely protocol-agnostic.

Supported transports:

- Serial (COM / tty)

- Other Transport (Future scope)

Responsibilities:

- Opens and configures hardware communication channels (COM / tty)

- Handles OS-specific serial I/O (Windows + Linux abstraction)

- Provides raw write/read byte streaming

- Buffers incoming data until complete lines (\n) are formed

- Ensures non-blocking / timeout-based reads

- Flushes stale or unwanted RX data when required

- Hides platform differences from upper layers

# 7. Multi-Protocol Support

Transport mechanisms differ in behavior:

| Transport | Command Based |
| --- | --- |
| Serial | Yes |
| CANopen | Object dictionary |
| EtherCAT | Process data |
| GPIO | Signal driven |

Internal adapters ensure the public API remains identical regardless of protocol.

# 8. Cross-Platform Strategy

Supported operating systems:

- Windows

- Linux

- macOS

Platform-specific implementations remain internal.

Example:

serial_win.cpp

serial_posix.cpp

Applications use a unified interface across all platforms.

# 9. Logging

The SDK provides minimal logging through a single macro:

XLOG(...)

Logging characteristics:

- compile-time enable/disable

- zero overhead when disabled

- no external dependencies

CMake option:

option(XERYON_ENABLE_LOG "Enable SDK logging" ON)

When disabled, XLOG compiles to no operation.

# 10. Extensibility

Adding a new controller requires:

- Controller implementation

- Command mapping

- Capability definition

Adding a new transport requires:

- Transport implementation

No public API modification is required.

# 11. Threading Model

Design assumptions:

- communication is non-blocking

- INFO processing is asynchronous

- applications may poll state safely

Thread management remains internal to the SDK.

# 12. Error Handling

Errors are represented using:

- status objects

- exceptions for fatal failures

- recoverable runtime errors when possible

Transport failures are isolated from application logic.

# 13. Build System

The SDK uses CMake.

Features:

- cross-platform builds

- optional logging

- installable package (TBD)

- examples and tests

Exported target:

xeryon::xeryon

# 14. Design Decisions Summary

| Decision | Reason |
| --- | --- |
| Axis abstraction | unify single and multi-axis controllers |
| Command-centered architecture | aligns with controller documentation |
| Transport abstraction | enables multi-protocol support |
| Controller implementations | scalable hardware support |
| Minimal logging | embedded-friendly design |
| Cross-platform first | unknown customer environments |

# 15. Future Extensions

Potential future additions:

- TBD

Architecture supports extension without redesign.

# 16. Guiding Principle

The SDK abstracts hardware complexity while preserving controller capability and performance.

# 17. Development Guidelines

## 17.1 Repository Structure

/sdk

/include        → Public headers

/src            → Core implementation

/controller     → Controller logic

/axis           → Axis abstraction

/communication  → Protocol implementations

/transport      → Hardware drivers

/logging        → XLOG implementation

/examples           → SDK usage examples

/tests              → Unit tests

/CMakeLists.txt     → Build configuration

Purpose:

- Prevents folder chaos later

- Helps onboarding instantly

## 17.2 Runtime Workflow

1. Application creates XController

2. Controller discovers hardware controllers

3. Axis objects are created

4. Commands issued through Command Model

5. Protocol layer encodes commands

6. Transport sends data

7. Feedback propagates upward

8. Shutdown releases resources

[Diagram: Runtime Execution Flow]

## 17.3 Versioning Policy

Versioning follows Semantic Versioning:

MAJOR  → Breaking API changes

MINOR  → Backward-compatible features

PATCH  → Bug fixes and internal improvements

Public API stability guaranteed within the same MAJOR version.