TwinCAT ENI — Configuration Reference

Xeryon drive on an EtherCAT segment — annotated configuration analysis

| Item | Value |
| --- | --- |
| Source file | TwinCAT_ENI.xml |
| Schema | EtherCATConfig.xsd, Version="1.3" (ETG.2100) |
| Contents | one EtherCAT master, one slave (Xeryon drive), one cyclic task |
| Purpose | A complete plain-language description of what this ENI configures, for use as a known-good reference when comparing against configurations generated by other masters. |

# Contents

# 1. Network identity

## Master

| Field | Value | Notes |
| --- | --- | --- |
| Name | Device 1 (EtherCAT) |  |
| Destination MAC | 01 01 05 01 00 00 | TwinCAT's standard EtherCAT destination |
| Source MAC | 4c ea 41 63 17 44 | the actual host NIC |
| EtherType | a488 | little-endian on the wire = 0x88A4, EtherCAT |

## Slave

| Field | Value | Notes |
| --- | --- | --- |
| Name | Box 1 (Drive) |  |
| AutoIncAddr | 0 | first physical node on the segment |
| PhysAddr | 1001 | station address assigned during Init→PreOp (0x03E9) |
| Physics | YY | both ESC ports are MII and in use |
| VendorId | 78 (0x4E) | Xeryon |
| ProductCode | 1 |  |
| RevisionNo | 1 |  |
| SerialNo | 0 | not checked |

The slave sits at auto-increment position 0, i.e. it is the first and only node, cabled directly to the host NIC. Entry is therefore via ESC port 0 by construction.

# 2. Process data

The drive uses a fixed PDO structure — both PDOs carry Fixed="true" and Mandatory="true". There is no CoE, no DS402, and no runtime PDO remapping. The mapping is baked into the ESI and cannot be changed by the master.

## Sync Managers

| SM | Direction | ESC address | Length | ControlByte | Enabled |
| --- | --- | --- | --- | --- | --- |
| SM0 | Outputs (master → drive) | 0x1000 | 20 bytes | 0x64 | yes |
| SM1 | Inputs (drive → master) | 0x1200 | 8 bytes | 0x20 | yes |

Only two Sync Managers are defined. There are no mailbox Sync Managers, which is consistent with a device that supports no CoE/EoE/FoE — the state machine goes straight to process data.

## FMMUs

| FMMU | Logical address | Length | Physical | Type |
| --- | --- | --- | --- | --- |
| FMMU0 | 0x01000000 | 20 bytes | 0x1000 | write (outputs) |
| FMMU1 | 0x01000000 | 8 bytes | 0x1200 | read (inputs) |

Both FMMUs share the same logical base address. That is intentional: a single LRW datagram writes 20 output bytes and reads 8 input bytes through the same logical window.

## Output map — RxPDO 0x1600 "Position" (20 bytes, SM0)

| Byte | Size | Field | Type |
| --- | --- | --- | --- |
| 0–3 | 32 bit | Command | STRING (4 ASCII chars) |
| 4–7 | 32 bit | Target position | DINT |
| 8–11 | 32 bit | Speed | UDINT |
| 12–13 | 16 bit | Acceleration | UINT |
| 14–15 | 16 bit | Deceleration | UINT |
| 16 | 8 bit | Execute | BYTE |
| 17–19 | 24 bit | padding | — |

The Command field is the Xeryon ASCII command word (DPOS, ENBL, INDX, SCAN, RSET, …). Execute is the handshake byte.

## Input map — TxPDO 0x1A00 "Status" (8 bytes, SM1)

| Byte.bit | Field |
| --- | --- |
| 0–3 | Actual position (DINT, 32 bit) |
| 4.0 | Amplifiers enabled |
| 4.1 | End stop |
| 4.2 | Thermal protection 1 |
| 4.3 | Thermal protection 2 |
| 4.4 | Force zero |
| 4.5 | Motor on |
| 4.6 | Closed loop |
| 4.7 | Encoder index |
| 5.0 | Encoder valid |
| 5.1 | Searching index |
| 5.2 | Position reached |
| 5.3 | Error compensation |
| 5.4 | Encoder error |
| 5.5 | Scanning |
| 5.6 | Left end stop |
| 5.7 | Right end stop |
| 6.0 | Error limit |
| 6.1 | Searching optimal frequency |
| 6.2 | Safety timeout |
| 6.3 | Execute ack |
| 6.4 | Emergency stop |
| 6.5 | Position fail |
| 6.6–6.7 | padding (2 bit) |
| 7 | Slot (BYTE) |

Execute ack at 6.3 is the acknowledge half of the Execute handshake.

# 3. Distributed Clocks

<DC> <ReferenceClock>true</ReferenceClock> <CycleTime0>1000000</CycleTime0> <!-- 1 000 000 ns = 1 ms --> <CycleTime1>0</CycleTime1> <ShiftTime>0</ShiftTime> </DC>

The drive is designated DC reference clock — unavoidable here, since it is the only slave.

DC is configured by four register writes, all on the PreOp→SafeOp transition:

| Register | Name | Value written | Decode |
| --- | --- | --- | --- |
| 0x09A0 | Sync0/Sync1 cycle time | 40 42 0f 00 00 00 00 00 | Sync0 = 0x000F4240 = 1 000 000 ns; Sync1 = 0 |
| 0x0990 | Start time cyclic operation | 00 00 00 00 00 00 00 00 | start immediately |
| 0x0980/0x0981 | Cyclic unit control / Activation | 00 03 | see below |
| 0x09A8 | Latch0 edge config | 00 00 | latch unused |

## The activation value

0x0980 = 0x00, 0x0981 = 0x03.

Register 0x0981 bit 0 is *cyclic operation enable*, bit 1 is *SYNC0 generation*. So 0x03 enables both. This corresponds to an ESI AssignActivate of #x0300 — SYNC0 pulses are generated in ESC hardware.

For reference, the other values encountered during commissioning:

| AssignActivate | 0x0981 | ENI Data | Meaning |
| --- | --- | --- | --- |
| #x0000 | 0x00 | 0000 | Sync unit disabled |
| #x0100 | 0x01 | 0001 | cyclic operation only, no SYNC0 pulses |
| #x0300 | 0x03 | 0003 | cyclic operation + SYNC0 pulses ← this file |

Because the drive firmware is polling-based and does not consume SYNC0, enabling the pulses is transparent to it; the sync unit runs entirely in the LAN9252 ESC hardware.

## Master-level DC preparation

Three broadcast writes on Init→PreOp set up the DC drift compensation loop:

| Register | Value | Purpose |
| --- | --- | --- |
| 0x0910 (32 bytes) | zeroed | clear system time area |
| 0x0930 | 0x1000 (4096) | speed counter start — standard default |
| 0x0934/0x0935 | filter depth 12 | system time / speed counter filter depth |

# 4. State machine — init command execution

<Transition> codes are ordered source→destination pairs: I=Init, P=PreOp, B=Bootstrap, S=SafeOp, O=Op. There is no code meaning "while in state X". The master walks its init command list on every transition and executes each entry whose transition list contains that transition. All 12 master commands carry BeforeSlave=1, so master broadcasts precede slave-specific commands within a transition.

Register shorthand used below: 0x0120 = AL Control, 0x0130 = AL Status, 0x0500 = EEPROM assignment, 0x0502/0x0508 = EEPROM control/data, 0x0010 = configured station address.

## Init → PreOp (IP) — master broadcasts

| # | Command | Purpose |
| --- | --- | --- |
| 1 | BRD 0x0130 ×2 | read slave count |
| 2 | BWR 0x0200 = 0x0004 | enable ECAT interrupt |
| 3 | BWR 0x0010 = 0 | clear configured station addresses |
| 4 | BWR 0x0300 (8 bytes) | clear CRC / error counters |
| 5 | BWR 0x0600 (256 bytes) | clear all FMMUs |
| 6 | BWR 0x0800 (256 bytes) | clear all Sync Managers |
| 7 | BWR 0x0910 (32 bytes) | clear DC system time |
| 8 | BWR 0x0981 = 0x00 | clear DC cyclic unit activation |
| 9 | BWR 0x0930, BWR 0x0934 | DC drift loop setup |
| 10 | BWR 0x0103 = 0x00 | disable the configured station alias (second physical address) |

## Init → PreOp (IP) — slave-specific

| # | Command | Purpose |
| --- | --- | --- |
| 11 | APWR 0x0120 = 0x01 | request Init |
| 12 | APRD 0x0130 | validate Init (0x01, mask 0x0F, timeout 3 000 ms) |
| 13 | APWR 0x0500 = 0x00 | assign EEPROM to ECAT |
| 14 | EEPROM reads at 0x0502/0x0508 | identity checks, 100 ms timeout each — VendorId word 0x0008 → 0x0000004E (78); ProductCode word 0x000A → 0x00000001; RevisionNo word 0x000C → ≥ 0x00000001 |
| 15 | APWR 0x0010 = 0x03E9 | set station address to 1001 |
| 16 | FPWR 0x0980 = 0x0000 | clear DC activation |
| 17 | FPWR 0x0800 (16 bytes) = 0 | clear Sync Managers |
| 18 | FPWR 0x0500 = 0x01 | hand EEPROM to PDI |
| 19 | FPWR 0x0120 = 0x02 | request PreOp |
| 20 | FPRD 0x0130 | validate PreOp (0x02, mask 0x1F, timeout 3 000 ms) |
| 21 | APWR 0x0500 = 0x00 | return EEPROM to ECAT |

The EEPROM handover at steps 13/18/21 matters: the master borrows the EEPROM interface to read the identity, then releases it to the PDI so the ESC/firmware can use it.

## PreOp → SafeOp (PS)

| # | Command | Purpose |
| --- | --- | --- |
| 1 | FPWR 0x09A0 | DC cycle time (1 ms) |
| 2 | FPWR 0x0990 | DC start time (0) |
| 3 | FPWR 0x0980 = 0x0003 | DC activation |
| 4 | FPWR 0x09A8 | latch config |
| 5 | FPWR 0x0800 | SM0: start 0x1000, length 20, control 0x64, enable |
| 6 | FPWR 0x0808 | SM1: start 0x1200, length 8, control 0x20, enable |
| 7 | FPWR 0x0600 | FMMU0: logical 0x01000000, 20 bytes, → 0x1000, write |
| 8 | FPWR 0x0610 | FMMU1: logical 0x01000000, 8 bytes, → 0x1200, read |
| 9 | FPWR 0x0120 = 0x04 | request SafeOp |
| 10 | FPRD 0x0130 | validate SafeOp (timeout 10 000 ms) |

DC is armed before the Sync Managers and FMMUs are configured, and the state request comes last.

## SafeOp → Op (SO)

| # | Command | Purpose |
| --- | --- | --- |
| 1 | FPWR 0x0120 = 0x08 | request Op |
| 2 | FPRD 0x0130 | validate Op (timeout 10 000 ms) |

Nothing touches DC. The 0x0981 = 0x03 written at PS is sticky and remains active through SafeOp and into Op.

## Downward transitions

| Transition | Meaning | DC action |
| --- | --- | --- |
| PP | PreOp refresh | clear 0x0980 |
| SP, OP | SafeOp/Op → PreOp | clear 0x0980, clear FMMUs, request PreOp |
| SI, OI | SafeOp/Op → Init | clear 0x0980 (auto-increment), clear FMMUs/SMs, request Init |
| OS | Op → SafeOp | request SafeOp only |

Note that OP means *Op → PreOp*, not "in Op". The three "clear DC activation" entries are all teardown paths.

Cold-start summary. On a clean boot the DC registers are touched exactly twice: cleared at IP, set at PS. Everything else is shutdown handling.

# 5. Cyclic frame

TaskId 3 Priority 20 CycleTime 1000 (µs → 1 ms) Comment synchronized with task 'PlcTask'

One frame, four datagrams, in transmission order:

| # | Cmd | Type | Target | Len | WKC | Active in |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | NOP | 0x0900 | 4 | — | PreOp, SafeOp, Op |
| 2 | 13 | ARMW | 0x0910 | 4 | — | PreOp, SafeOp, Op |
| 3 | 12 | LRW | logical 0x01000000 | 20 | 3 | SafeOp, Op |
| 4 | 7 | BRD | 0x0130 | 2 | 1 | PreOp, SafeOp, Op |

Datagram 2 is the mechanism that keeps DC locked. ARMW (Auto-increment Read Multiple Write) reads the system time from the reference clock slave and writes it to every slave behind it, once per cycle. It is declared active in Op, which is what keeps 0x0981 = 0x03 meaningful during operation.

Datagram 3 carries the process data: 20 output bytes written, 8 input bytes read, through the shared logical window. Expected working counter is 3 — LRW increments WKC by 1 for the read and 2 for the write on a slave that does both.

Datagram 1 is declared as command type 0 (NOP) targeting 0x0900, the ESC receive-time latch register. A NOP does not itself latch anything, so this occupies a reserved slot in the frame rather than performing the write its position implies. Worth confirming on the wire what the master actually emits here if precise DC delay measurement ever matters.

# 6. TwinCAT process image

The <ProcessImage> section maps PDO entries to TwinCAT variables. Both directions declare ByteSize=1536.

Process data begins at bit offset 464 (byte 58) in both directions. Everything at offsets above bit 12160 is TwinCAT-internal diagnostics and never appears on the wire:

- Frm0State, Frm0WcState, Frm0InputToggle — frame-level status

- WcState — working counter validity per slave and per sync unit

- SlaveCount, CfgSlaveCount, DevState, ChangeCount, DevId, AmsNetId

- Box 1 (Drive).InfoData.DcOutputShift / DcInputShift — DC shift diagnostics

DcOutputShift and DcInputShift are worth watching during commissioning; they report the actual applied shift and will expose a mismatch between the declared cycle and reality.

# 7. Observations and anomalies

## ESI defects visible in the PDO map

The TxPDO entry list has two flaws inherited from the ESI:

- SubIndex 4 is used twice — both "Thermal protection 1" and "Thermal protection 2" declare SubIndex 4; SubIndex 5 is never used.

- All entries share one object index — every TxPDO entry declares Index #x1 and every RxPDO entry Index #x2, rather than each entry pointing at its own object.

Neither affects bit packing, since the master lays entries out sequentially by BitLen and the totals are correct (64 input bits, 160 output bits). Both would fail a strict ETG conformance check, and the duplicate SubIndex will confuse any tool that keys entries by index/subindex.

## Padding is explicit

Both PDOs end with an unnamed entry (2 bits input, 24 bits output) to pad to a byte boundary. This is correct practice and makes the 8- and 20-byte SM lengths exact.

## No mailbox

The absence of mailbox Sync Managers is the structural consequence of the drive having no CoE. Any master expecting to run a DS402 startup sequence, read object 0x1000, or perform SDO-based PDO assignment will fail against this device. It must be treated as a generic EtherCAT slave with a fixed process image.

# 8. Comparison checklist

When diffing this against an ENI generated by another master, these are the values that determine whether DC behaves identically:

| Item | Value here |
| --- | --- |
| PS write to 0x0980 | 0003 |
| 0x0990 start time | 0 (8 bytes) |
| 0x09A0 Sync0 cycle | 0x000F4240 (1 ms) |
| 0x09A8 latch config | 0000 |
| <CycleTime0> | 1000000 ns |
| <ShiftTime> | 0 |
| <ReferenceClock> | true |
| Cyclic ARMW at 0x0910 | active in PreOp, SafeOp, Op |
| <Cyclic><CycleTime> | 1000 (µs per ETG.2100) |
| SM0 / SM1 | 0x1000/20/0x64, 0x1200/8/0x20 |
| Identity | VendorId 78, ProductCode 1, RevisionNo ≥ 1 |

The <Cyclic><CycleTime> unit is the one to watch: ETG.2100 specifies microseconds, so 1000 here is consistent with the 1 ms CycleTime0. A master that emits 1 for the same physical cycle is either using milliseconds internally or is genuinely misdeclaring the frame rate by a factor of 1000.