# Xeryon EtherCAT Slave — Full State Lifecycle

TwinCAT Master — Init → PreOp → SafeOp → Op

Source captures:

- scan_one_slave_270726.pcapng — idle network + device scan + Init → PreOp

- scan_one_slave_preop_to_op_270726.pcapng — PreOp → SafeOp → Op + steady-state cyclic exchange

Slave: single Xeryon EtherCAT slave, no CoE/DS402 mailbox, proprietary PDO structure.

Fixed station address assigned during scan: 0x03E9 (1001 decimal).

# Phase 0 — Idle

The first ~37 s of the scan capture is background LAN traffic (SSDP, mDNS, DHCPv6) captured before the TwinCAT scan was triggered. No EtherCAT activity in this window.

# Phase 1 — Init: reset & port configuration (broadcast)

t ≈ 37.17 s. All commands are broadcast (BWR/BRD, ADP 0x0000, WC confirms exactly 1 slave present):

| Register | Cmd | Value | Meaning |
| --- | --- | --- | --- |
| 0x0101 (DL Control) | BWR | 0x00 | All 4 ports → Auto Loop |
| 0x0130 (AL Status) | BRD | 0x0001 | INIT confirmed, error = false |
| 0x0200 (ECAT IRQ Mask) | BWR | 0x0004 | Enable ESC Status Event interrupt |
| 0x0010 (Station Alias) | BWR | 0x0000 | Cleared — still position-addressed |
| 0x0300–0x0307 (CRC/RX err.) | BWR | 0x00 x4 | Cleared on all 4 ports |
| 0x0600, 256B (16 FMMUs) | BWR | all zero | FMMU table cleared/deactivated |
| 0x0800, 256B (8 SMs) | BWR | all zero | SM table cleared/disabled |
| 0x0910, 32B (DC time/offset/delay) | BWR | all zero | DC state cleared |
| 0x0981 (DC Activation) | BWR | 0x00 | Sync0/Sync1 off |
| 0x0930 (DC Speed Ctr Start) | BWR | 0x1000 | DC speed-counter seed |
| 0x0934/0x0935 (DC filter depths) | BWR | 0x00 / 0x0c | Sys-time-diff filter=0, speed filter=12 |
| 0x0103 (DL Control byte 3) | BWR | 0x00 | Reserved/PDI bits |

This is a clean-slate reset applied to every slave found — nothing device-specific yet.

# Phase 2 — Init: identification (position-addressed)

- APRD 0x0000 — Type register read

EEPROM identification via APWR 0x0502 (set word address) + APRD 0x0508 (read data word):

| EEPROM word addr | Value read | Field |
| --- | --- | --- |
| 0x0008 | 0x004E | Vendor ID (low word) |
| 0x000A | 0x0001 | Product Code (low word) |
| 0x000C | 0x0001 | Revision Number (low word) |
| 0x000E | (read once) | Serial Number (low word) |

TwinCAT uses this to match the slave against its known device/ESI database.

# Phase 3 — Init: fixed address assignment

APWR 0x0010 = 0x03E9 — station address 1001 assigned. From this point on, the slave is addressed with FPRD/FPWR to 0x03E9 instead of positional addressing.

# Phase 4 — Init → PreOp request (retried 3×)

Each of 3 passes repeats broadcast reset → EEPROM ID → address assignment, then:

- APWR AL Control = 0x0001 (Init request, broadcast/position)

- FPWR AL Control = 0x0002 (PreOp request, fixed address 0x03E9)

- FPRD AL Status polled after each write

On the 3rd pass (t ≈ 37.91 s), AL Status finally reads back 0x0002 (PreOp), AL Status Code 0x0000 (no error), and stays there — no further AL Control writes follow.

No AL Status Code error was ever set during any pass. The 3 repeats look like TwinCAT's conservative re-verification behavior, not a fault.

# Phase 5 — PreOp steady state

From t ≈ 37.91 s to the end of capture 1, and continuing into the start of capture 2:

- FPRD 0x0130 (AL Status) and FPRD 0x0300 (watchdog/CRC counters) polled every ~100 ms

- AL Status stays 0x0002 (PreOp), error = false throughout

# Phase 6 — PreOp → SafeOp: mailbox & process-data setup

t ≈ 1.35–1.37 s in capture 2.

## Sync Managers (FPWR, 8 bytes each, fixed address 0x03E9)

| SM | Register | Start | Length | Access | Watchdog | Enable |
| --- | --- | --- | --- | --- | --- | --- |
| SM0 (mbx out) | 0x0800 | — | — | — | — | Disabled (no CoE mailbox) |
| SM1 (mbx in) | 0x0808 | — | — | — | — | Disabled |
| SM2 (outputs) | 0x0810 | 0x1000 | 20 B | Write | Enabled | True |
| SM3 (inputs) | 0x0818 | 0x1200 | 8 B | Read | Off | True |

## FMMUs (FPWR, 16 bytes each)

| FMMU | Register | Logical Start | Length | Phys Start | Type | Active |
| --- | --- | --- | --- | --- | --- | --- |
| FMMU0 (outputs) | 0x0600 | 0x01000000 | 20 B | 0x1000 | Write | Yes |
| FMMU1 (inputs) | 0x0610 | 0x01000000 | 8 B | 0x1200 | Read | Yes |

Both FMMUs share the same logical start address because the slave is exchanged with a single combined LRW datagram (read + write in one pass) rather than separate LRD/LWR commands — the write half lands on FMMU0/phys 0x1000, the read half on FMMU1/phys 0x1200, both within the same 20-byte logical window.

This device has no mailbox channel at all — SM0/SM1 stay disabled for the device's entire life. All communication runs through the two process-data Sync Managers (SM2 outputs / SM3 inputs) and their FMMUs. Only SM2 (outputs) has the local watchdog-trigger bit set — protecting against a stale/missing write from the master by falling back safely on the slave side.

# Phase 7 — SafeOp request/confirm + cyclic start

| Time | Event |
| --- | --- |
| t = 1.3675 s | FPWR AL Control = 0x0004 (SafeOp requested) |
| t = 1.3715 s | FPRD AL Status = 0x0004, error = false — confirmed in ~4 ms |

The instant SafeOp is confirmed, cyclic LRW datagrams (20-byte logical read-write) begin, each frame also carrying a piggy-backed BRD 0x0130 AL Status check.

# Phase 8 — Op request/confirm

| Time | Event |
| --- | --- |
| t = 1.3725 s | FPWR AL Control = 0x0008 (Op requested) — one cycle after SafeOp confirmed |
| t = 1.3764 s | FPRD AL Status = 0x0008, error = false — confirmed in ~4 ms |

# Phase 9 — Operational steady state

- 2350 cyclic LRW exchanges over the remainder of the capture

- Working counter = 3 on every single cycle — zero drops, zero WC mismatches

- Average cycle period ≈ 2.04 ms

- AL Status checked every cycle, stays 0x0008 (Op) throughout, error = false

# Summary

| Milestone | Outcome |
| --- | --- |
| Slaves found | 1 (confirmed via WC and ADP auto-increment on broadcast) |
| Fixed address | 0x03E9 |
| Mailbox (CoE) | Not present — SM0/SM1 disabled for life of device |
| Process data | SM2 (out, 20B, watchdog-guarded) / SM3 (in, 8B) via FMMU0/FMMU1 |
| Init → PreOp | 3 conservative retries, ~4.7 s wall clock (no errors) |
| PreOp → SafeOp → Op | < 10 ms once mailbox/SM/FMMU were configured |
| Cyclic exchange | 2350 cycles, WC = 3 throughout, ~2.04 ms period, zero faults |

No error codes or working-counter faults were observed anywhere across either capture.